Chapter 1: Introducing Hardware

TRUE/FALSE

1.	A serial port transmits data in parallel and is most often used by an older printer.

ANS:	F	PTS:	1	REF:	9

2.	When data and instructions are not being used, they must be kept in permanent storage, sometimes called primary storage.

ANS:	F	PTS:	1	REF:	10

3.	Parallel ATA, sometimes called the EIDE (Enhanced IDE) standard or the IDE standard, is slower than SATA.

ANS:	T	PTS:	1	REF:	15

4.	A motherboard can have more than one bus, each using a different protocol, speed, data path size, and so on.

ANS:	T	PTS:	1	REF:	20

5.	Some very basic instructions are stored on the motherboard—just enough to start the computer, use some simple hardware devices such as a monitor and keyboard, and search for an operating system stored on a storage device such as a hard drive or CD.

ANS:	T	PTS:	1	REF:	26

MULTIPLE CHOICE

1.	In the world of computers, the term ____ refers to the computer’s physical components, such as the monitor, keyboard, motherboard, and hard drive.

| a. | software | c. | architecture |
| --- | --- | --- | --- |
| b. | middleware | d. | hardware |

ANS:	D	PTS:	1	REF:	2

2.	The term ____ refers to the set of instructions that directs the hardware to accomplish a task.

| a. | software | c. | middleware |
| --- | --- | --- | --- |
| b. | hardware | d. | stack |

ANS:	A	PTS:	1	REF:	2

3.	To perform a computing task, software uses hardware for four basic functions: ____.

| a. | input, storage, retrieval, and display | c. | input, storage, alteration, and output |
| --- | --- | --- | --- |
| b. | input, processing, storage, and output | d. | output, input, analysis, and viewing |

ANS:	B	PTS:	1	REF:	2

4.	A 1 or 0 in this system is called a ____.

| a. | byte | c. | datum |
| --- | --- | --- | --- |
| b. | word | d. | bit |

ANS:	D	PTS:	1	REF:	3

5.	Most input/output devices communicate with components inside the computer case through a wireless connection or through cables attached to the case at a connection called a(n) ____.

| a. | port | c. | interface |
| --- | --- | --- | --- |
| b. | slot | d. | socket |

ANS:	A	PTS:	1	REF:	4

6.	The printer produces output on paper, often called ____ copy.

| a. | real | c. | hard |
| --- | --- | --- | --- |
| b. | soft | d. | virtual |

ANS:	C	PTS:	1	REF:	6

7.	A device that is not installed directly on the motherboard is called a(n) ____ device.

| a. | standard | c. | extraneous |
| --- | --- | --- | --- |
| b. | peripheral | d. | perimeter |

ANS:	B	PTS:	1	REF:	8

8.	A ____ bus port can be used by many different input/output devices, such as keyboards, printers, scanners, and digital cameras.

| a. | unified serial | c. | universal serial |
| --- | --- | --- | --- |
| b. | universal static | d. | universal standards |

ANS:	C	PTS:	1	REF:	9

9.	The ____ is a group of microchips on the motherboard that control the flow of data and instructions to and from the processor.

| a. | chipset | c. | bridge |
| --- | --- | --- | --- |
| b. | block | d. | gate |

ANS:	A	PTS:	1	REF:	9

10.	Primary storage is provided by devices called memory or ____ located on the motherboard and on some adapter cards.

| a. | ROM | c. | Flash |
| --- | --- | --- | --- |
| b. | BIOS | d. | RAM |

ANS:	D	PTS:	1	REF:	11

11.	Most ____ drives consist of a sealed case containing platters or disks that rotate at a high speed.

| a. | optical | c. | flash |
| --- | --- | --- | --- |
| b. | hard | d. | solid state |

ANS:	B	PTS:	1	REF:	13

12.	A(n) ____ drive is considered standard equipment on most computer systems today because most software is distributed on CDs or DVDs.

| a. | floppy | c. | optical |
| --- | --- | --- | --- |
| b. | flash | d. | hard |

ANS:	C	PTS:	1	REF:	16

13.	USB ____ drives are compact, easy to use, and currently hold up to 64 GB of data.

| a. | worm | c. | hard |
| --- | --- | --- | --- |
| b. | optical | d. | flash |

ANS:	D	PTS:	1	REF:	17

14.	A(n) ____ is a set of rule and standards that any two entities use for communication.

| a. | protocol | c. | interface |
| --- | --- | --- | --- |
| b. | language | d. | interaction |

ANS:	A	PTS:	1	REF:	19-20

15.	The ____ or system timer is dedicated to timing the activities of the chips on the motherboard.

| a. | randomizer | c. | system clock |
| --- | --- | --- | --- |
| b. | ALU | d. | interrupt manager |

ANS:	C	PTS:	1	REF:	20

16.	Everything in a computer is ____.

| a. | decimal | c. | hexadecimal |
| --- | --- | --- | --- |
| b. | binary | d. | integers |

ANS:	B	PTS:	1	REF:	21

17.	The ____ card, also called a graphics card, provides one or more ports for a monitor.

| a. | video | c. | input |
| --- | --- | --- | --- |
| b. | interface | d. | output |

ANS:	A	PTS:	1	REF:	23

18.	The ____ card provides a port for a network cable to connect the PC to a network.

| a. | video | c. | modem |
| --- | --- | --- | --- |
| b. | interface | d. | network |

ANS:	D	PTS:	1	REF:	23

19.	The most important component of the computer’s electrical system is the ____, which is usually near the rear of the case.

| a. | power supply | c. | drive cage |
| --- | --- | --- | --- |
| b. | cooling fan | d. | power bus |

ANS:	A	PTS:	1	REF:	25

20.	Data and instructions are stored on special ROM (read-only memory) chips on the board and are called the ____.

| a. | flash | c. | BIOS |
| --- | --- | --- | --- |
| b. | microcode | d. | Symbols |

ANS:	C	PTS:	1	REF:	26

21.	Motherboard settings are stored in a small amount of RAM located on the firmware chip and are called ____.

| a. | CMOS ROM | c. | DMOS ROM |
| --- | --- | --- | --- |
| b. | CMOS RAM | d. | HMAC RAM |

ANS:	B	PTS:	1	REF:	26

COMPLETION

1.	A(n) ___________________ is a circuit board that holds microchips, or integrated circuits (ICs), and the circuitry that connects these chips.

ANS:	adapter card

PTS:	1	REF:	7

2.	RAM memory is called ____________________ because it is temporary in nature.

ANS:	volatile

PTS:	1	REF:	11

3.	____________________ are circuits or paths that enable data, instructions, and power to move from component to component on the board.

ANS:	Traces

PTS:	1	REF:	19

4.	The size and shape of an expansion slot depend on the kind of ____________________ it uses.

ANS:	bus

PTS:	1	REF:	21

5.	Software embedded into hardware is often referred to as ____________________ because of its hybrid nature.

ANS:	firmware

PTS:	1	REF:	26

MATCHING

Match each term with the correct statement below.

| a. | Monitor | f. | Hertz |
| --- | --- | --- | --- |
| b. | CPU | g. | Keyboard |
| c. | Megahertz | h. | Gigahertz |
| d. | Flash ROM | i. | Motherboard |
| e. | Mouse |  |  |

1.	Writes data and instructions in storage devices and performs calculations and other data processing

2.	The primary input device of a computer

3.	Pointing device used to move a pointer on the screen and to make selections

4.	The visual device that displays the primary output of the computer

5.	Contains a socket to hold the CPU

6.	One cycle per second

7.	One million cycles per second

8.	One billion cycles per second

9.	Non-volatile memory and can be reprogrammed

1.	ANS:	B	PTS:	1	REF:	4

2.	ANS:	G	PTS:	1	REF:	4

3.	ANS:	E	PTS:	1	REF:	4

4.	ANS:	A	PTS:	1	REF:	6

5.	ANS:	I	PTS:	1	REF:	8

6.	ANS:	F	PTS:	1	REF:	21

7.	ANS:	C	PTS:	1	REF:	21

8.	ANS:	H	PTS:	1	REF:	21

9.	ANS:	D	PTS:	1	REF:	27

SHORT ANSWER

1.	Describe the binary number system.

ANS:

The number system that only uses two digits, 0 and 1, is called the binary number system. In a computer, all counting and calculations use the binary number system. Because of the way the number system is organized, grouping is often done in groups of eight bits, each of which is called a byte. Counting in binary goes like this: 0, 1, 10, 11, 100, 101, and so forth. For example, in binary code the number 25 is 0001 1001. When text is stored in a computer, every letter or other character is first converted to a code using only zeros and ones. The most common coding method for text is ASCII (American Standard Code for Information Interchange).

PTS:	1	REF:	3

2.	List and describe the elements that each hardware input, output, or storage device requires.

ANS:

A method for the CPU to communicate with the device. The device must send data to and/or receive data from the CPU. The CPU might need to control the device by passing instructions to it, or the device might need to request service from the CPU.

Software to instruct and control the device. A device is useless without software to control it. The software must know how to communicate with the device at the detailed level of that specific device, and the CPU must have access to this software in order to interact with the device. Each device responds to a specific set of instructions based on the device’s functions. The software must have an instruction for each possible action you expect the device to accomplish.

Electricity to power the device. Electronic devices require electricity to operate. Devices can receive power from the power supply inside the computer case, or they can have their own power supplied by a power cable connected to an electrical outlet.

PTS:	1	REF:	4

3.	List three devices that are contained inside the computer case.

ANS:

A motherboard containing the CPU, memory, and other components

A hard drive and optical drive (CD or DVD) used for permanent storage

A power supply with power cords supplying electricity to all devices inside the case

Adapter cards used by the CPU to communicate with devices inside and outside the case

Cables connecting devices to adapter cards and the motherboard

PTS:	1	REF:	7

4.	List three components that allow the processor to communicate with other devices.

ANS:

Traces, or wires, on the motherboard used for communication

Expansion slots to connect expansion cards to the motherboard

The system clock that keeps communication in sync

Connections for data cables to devices inside the case

Ports for devices outside the case

PTS:	1	REF:	8

5.	Describe the purpose of the chipset.

ANS:

The processor or CPU is the chip inside the computer that performs most of the actual data

processing. The processor could not do its job without the assistance of the chipset, a group of microchips on the motherboard that control the flow of data and instructions to and from the processor. The chipset is responsible for the careful timing and coordination of activities. The chipset is an integrated component of the motherboard and is contained in two packages embedded on the motherboard.

PTS:	1	REF:	9

6.	Describe the two major ATA standards for drive interface.

ANS:

The two major ATA standards for a drive interface are serial ATA (SATA), the newer and faster standard, and parallel ATA (PATA), the older and slower standard. Hard drives, CD drives, DVD drives, Zip drives, and tape drives, among other devices, can use these interfaces.

PTS:	1	REF:	13-14

7.	Describe solid state devices.

ANS:

A newer technology for hard drives uses nonvolatile flash memory chips, rather than using moving mechanical disks, to hold the data. These flash memory chips are similar to those used in USB flash drives. Any device that has no moving parts is called solid state (solid parts versus moving parts). Therefore, a drive made with flash memory is called a solid state drive (SSD), solid state disk (SSD), or solid state device (SSD). (Unfortunately, the acronym can have either definition).

PTS:	1	REF:	13

8.	Describe a floppy disk drive.

ANS:

An older secondary storage device sometimes found inside the case is a floppy drive, also called a floppy disk drive (FDD), that can hold 3.5-inch disks containing up to 1.44 MB of data.

PTS:	1	REF:	18

9.	Where are expansion cards mounted?

ANS:

Expansion cards are mounted in expansion slots on the motherboard.

PTS:	1	REF:	23

10.	What is the easiest way to determine the function of a particular expansion card?

ANS:

The easiest way to determine the function of a particular expansion card (short of seeing its name written on the card, which doesn’t happen very often) is to look at the end of the card that fits against the back of the computer case. A network card, for example, has a port designed to fit the network cable. A modem card has one, or usually two, telephone jacks as its ports. However, as you examine the ports on the back of your PC, remember that sometimes the motherboard provides ports of its own.

PTS:	1	REF:	25